CellExplorer: a graphical user interface and a standardized pipeline for visualizing and characterizing single neurons

Peter C. Petersen1, Josh Siegle4, Nick Steinmetz5, Sara Mahallati6, György Buzsáki1, 2, 3

Affiliations

1Neuroscience Institute and 2Department of Neurology, Langone Medical Center, New

York University, New York, NY 10016, USA

3Center for Neural Science, New York University, New York, NY 10003, USA

4Allen Institute for Brain Science, 615 Westlake Avenue North, Seattle, 98109, Washington, USA

5Department of Biological Structure, University of Washington, Seattle, WA 98195, USA

6 Institute of Biomedical Engineering, Krembil Research Institute, University of Toronto, Toronto, ON M5T 1M8, Canada

*Correspondence: gyorgy.buzsaki@nyumc.org & petersen.peter@gmail.com

Abstract

The large diversity of neuron types of the brain, characterized by a unique set of electrophysiological characteristics, provides the means by which cortical circuits perform complex operations. To quantify, compare, and visualize the functional features of single neurons, we have developed the open-source framework CellExplorer. It consists of three components: a processing module that extracts and calculates a standardized set of physiological metrics, including the classification of cell types and the determination of monosynaptic connections, a flexible data structure, and a powerful graphical interface. The graphical interface allows for the exploration of the computed features of neurons in any combination at the speed of a mouse click. The CellExplorer framework allow users to relate their data to a growing selection of ground truth cell types as well as to tens of thousands of single neurons collected across our labs. We believe CellExplorer will accelerate the linking of physiological properties of single neurons in the intact brain to genetically identified types.

INTRODUCTION

Discovering novel mechanisms in brain circuits requires high-resolution monitoring of the constituent neurons and understanding the nature of their interactions. Identification and manipulation of different neuron types in the behaving animal is a prerequisite to decipher their role in circuit dynamics and behavior. Yet, currently, a large gap exists between neuron classification schemes based on molecular and physiological methods (Kepecs and Fishell, 2014; Klausberger and Somogyi, 2008; McBain and Fisahn, 2001; Roux and Buzsáki, 2015; Rudy et al., 2011).

The large-scale single-unit recording method asserts that the relationship between neuronal firing and behavioral and cognitive variables can provide insights about the computational role of neurons and neuronal assemblies (Barlow, 1972; Buzsáki, 2004). However, exploiting the power of correlations between neuronal firing and behavioral variables requires multiple-level characterization of single neurons and their interactions along with numerous controls to exclude the contributions of many potential hidden variables. Ideally, neuron features need to be described at multiple levels of complexity. Long-term simultaneous recordings from large numbers of neurons allow the building of an extensive list of neuron properties (Fig. 1). The first level is a description of the biophysical and physiological characteristics of single neurons. This step includes waveform features, their physical position relative to recording sites, and other units (Csicsvari et al., 2003), interspike intervals statistics, and autocorrelograms. These first-level parameters are ‘stable’ features and can be used to combine data sets across animals both within and across laboratories for the first-order separation of single cells into putative major classes, typically excitatory and inhibitory cells. The second level relates single cells to other neurons and includes cross-correlations, monosynaptic connections, relationship to multiple oscillatory and irregular local field potentials (LFP), and unit population patterns. Additionally, it can describe long-term firing rates in defined brain states. The third level descriptors of single-unit activity, ideally, should include the relationship between its firing patterns and overt behavioral correlates, including spontaneous motor patterns and autonomic parameters (McGinley et al., 2015; Steinmetz et al., 2019). Verification and refinement of these properties can be assisted by optogenetic methods, which can relate physiological parameters to genetically identified neuron groups (Boyden et al., 2005; Klausberger and Somogyi, 2008; Rudy et al., 2011; Buzsáki et al., 2015; Roux and Buzsáki, 2015). Antidromic and unit-LFP coupling techniques provide further assignment of single neurons to cortical regions, layers, and target projections (Ciocchi et al., 2015; Senzai et al., 2019; Zhang et al., 2013).

This three-level description provides universal features of neuronal activity common to all experimental paradigms and, therefore, is communicable across different experiments and laboratories. In turn, these universal features can be contrasted and compared with higher-level or hidden correlates, such as learning, memory, decision making, emotions, and social interactions. Because these inferred high-level correlates of neuronal activity are often paradigm-specific and may differ across laboratories, the three-level analysis can guard against mistakenly assigning cognitive roles of neuronal spiking that may be explained by measurable overt correlates. Yet, even if all of the above information is available separately, factoring out critical variables and their combinations is possible only when the multitudes of single neuron characteristics can be compared flexibly.

Figure 1: Multifaceted single neuron characterization A. Using high-density silicon probes or multiple tetrodes (shown is a single shank with 8 recording sites), dozens to hundreds of neurons can be recorded simultaneously. B, Spikes of putative single neurons are extracted from the recorded traces and assigned to individual neurons through spike sorting algorithms and assigned to recording sites reflecting the neurons’ depth position in the brain (representation shows neurons projected on a silicon probe with 6 shanks). Features of the red neurons are shown in E-G. C. Neuron types are separated by first-order physiological parameters. D. Optogenetic and other direct methods can ground units to neuron types.  E. Single neurons are further characterized by their monosynaptic connections to other neurons. F. Connection vector of converging pyramidal cells to a single interneuron in a low dimensional representation (t-SNE) from physiological and functional features. G. Relating spikes to LFP patterns. H-I. Spike pattern correlations with overt behaviors. J, K. Spike correlations with experimenter-presented stimuli or situations. L, Spike-control of body extensions and robots. M. Spike correlates of cognition. A to D, first-level descriptors. E-G, second-level descriptors. H, I. third-level descriptors. J-M. Spike patterns related to inferred variables.

Whether testing a specific hypothesis or data mining of ever-growing data sets for discovery, the process can be advanced by fast and user-friendly visualization methods that facilitate efficient hypothesis testing. Toward this goal, we developed the open-source MATLAB based framework, CellExplorer, to characterize and classify single-cell features from multi-site extracellular recordings. It consists of a pipeline for extracting and calculating physiological features, a flexible data format, and a powerful graphical interface that allows for fast manual curation and feature exploration.

RESULTS

The CellExplorer architecture and operation consist of three main parts: a processing module for feature extraction, a graphical interface for manual curation and exploration, and a standardized yet flexible data structure (Fig 2). A step-by-step tutorial is available in the supplementary section, and more tutorials are available online (Suppl. Video 1). The first step in running the pipeline is defining the data input.

Figure 2. Three-component framework. Single yet extensive processing module (green); Standardized yet flexible data structure (yellow); and Graphical interface (purple). Data inputs are compatible with most existing spike sorting algorithms (grey). The data structure joins the Processing module with the Graphical interface (* signifies data containers). CellExplorer is open-source, built-in MATLAB, and available on GitHub.

Data Input

Before running the pipeline, relevant metadata should be defined describing the spike format, raw data, and experimental metadata (Fig. 2). All experimental metadata is handled in a single MATLAB structure, as part of the data structure, with a GUI for manual entry. The platform supports several spike sorting data formats, including Neurosuite, Phy, KiloSort, SpyKING Circus, Wave_Clus, MClust, AllenSDK, NWB, ALF, MountainSort, IronClust, (Chung et al., 2017; Hazan et al., 2006; Pachitariu et al., 2016; Quiroga et al., 2004; Schmitzer-Torbert et al., 2005; Yger et al., 2018). The raw data (wide-band) is critical for comparing derived metrics across laboratories since preprocessed data vary across laboratories and depend on equipment type and filter settings. Yet, for many applications, restricted to the examination of the unit–behavior relationship and neuronal spike-interaction analysis, entering metadata and spike timestamps are sufficient.

Processing Module

From the input data, the processing module will generate a large battery of spike features corresponding to a 3-level description of neuronal firing and their relationship to experiment-specific behaviors (Figure 2; Suppl. Table 2). The processing module is comprised of a single MATLAB script ProcessCellMetrics.m, which computes metrics using a modular structure. The first level description provides temporal features, waveform features (filtered and wideband), interspike interval statistics (ISIs), and autocorrelograms (ACGs). Next, the unit parameters are used for the initial classification of single cells into broad classes, default into putative pyramidal cells, narrow waveform interneurons, and unclassified cells. In experiments with silicon probes, the physical position relative to recording sites is also determined using trilateration (Petersen and Berg, 2016; Csicsvari et al., 2003).

The second level of description relates single neuron spikes to the activity of other neurons and population patterns. These metrics include spike cross-correlograms (CCGs), quantitative identification of putative monosynaptic connections, phase relationships to various oscillations, and irregular local field potentials (LFP) and unit population patterns. Monosynaptic connections, in turn, can be used to physiologically identify putative excitatory and inhibitory interneurons and use this information to refine the primary unit classification (Fig. 1E; Suppl Fig. 3H) (Barthó et al., 2004; English et al., 2017). All parameters can be customized according to the needs of each experimental paradigm (Suppl Table 2; CellExplorer.org/datastructure/standard-cell-metrics).

The third level descriptors are used to assess the relationship between firing patterns of neurons and overt behaviors, including immobility, locomotion, and running speed. Level 1-3 descriptors can be then further refined by optogenetic methods, which can relate physiological parameters to genetically identified neuron groups and justify or modify the primary cell type classification (Boyden et al., 2005; Buzsáki et al., 2015; Roux and Buzsáki, 2015). When available, antidromic and unit-LFP coupling techniques provide additional information about single neurons, such as their position in cortical regions, layers, and their target projections (Ciocchi et al., 2015; Senzai et al., 2019; Zhang et al., 2013). Because these 3-level descriptors of single unit features are universal, they can be readily compared with similar analyses across laboratories, independent of paradigm-specific features. The Processing Module automatically generates all cell metrics in a standardized fashion.

Figure 3: Graphical interface. A. The interface consists of 4 to 9 main plots, where the top row is dedicated to population-level representations of the neurons. Other plots are selectable and customizable for individual cells (e.g., single waveforms, ACGs, ISIs, CCGs, PSTHs, response curves, and firing rate maps). The surrounding interface consists of panels placed on either side of the graphs. The left side displays settings and population settings, including a custom plot panel, color group panel, display settings panel, and legends. The right side-panel displays single-cell dimensions, including a navigation panel, cell assignment panel, tags, and a table with metrics. In addition, there are text fields for a custom text filter and a message log. B. Layout examples highlighting three configurations with 1-3 group plots and 3-6 single cell plots. C. The interface has many interactive elements, including navigation and selection from plots (left mouse click links to selected cell and right mouse click selects the cell from all the plots), visualization of monosynaptic connections, various data plotting styles (more than 30+ unique plots built-in), supports custom plots; plotting filters can be applied by text or selection, keyboard shortcuts, zooming any plot by mouse-scrolling and polygon selection of cells. D. Group plotting options: 2D, 3D, raincloud plot, t-SNE, and double histogram. Each dimension can be plotted on linear or logarithmic axes. E. Single-cell plot options: waveform, ACG, ISI, firing rate across time, PSTH, response curve, firing rate maps, neuron position triangulation relative to recording sites, and monosynaptic connectivity graph. F. Most single-cell plots have three representations: individual single-cell representation, single-cell together with the entire population with absolute amplitude and a normalized image representation (colormap).

Data structure

The data structure (summarized in Fig. 2, and supplementary Fig. 1), is structured into data categories (containers) and MATLAB structures, which functionally separate related data, making them easily interpretable (human-readable) but also makes them machine-readable. The format is derived from the Buzcode (a MATLAB based data format for electrophysiological recordings and toolset developed communally in the Buzsaki lab; github.com/buzsakilab/buzcode), Neurosuite (neurosuite.sourceforge.net), and the Freely Moving Animal (FMA) Toolbox

(fmatoolbox.sourceforge.net).

The two most important structures are the session metadata struct and cell_metrics struct.

session metadata struct: Contains all session-level experimental metadata. It is a modular structure that makes it flexible and expandable, intuitive and interpretable, and it offers a single structure preventing scattering of metadata. A GUI allows for intuitive manual metadata entry, and a template script can assist in importing experimental metadata.

cell_metrics struct: Modular structure containing all metrics calculated in the processing module. It consists of three types of data-fields for handling the diverse types of data: numeric double, character-cells, and structs. Single value metrics are stored in double and character cells for respective numeric and character metrics. Time series (e.g., waveforms), group data (e.g. synaptic connections), and session parameters are stored in predefined struct modules. This structure makes the fields machine-readable, including user-defined metrics, providing expandability and flexibility, yet maintaining compatibility with the graphical interface. The single struct allows for processing multiple sessions together in the graphical interface (batch processing) and is convenient for sharing with collaborators and the broader scientific community in publications (see Supplementary Section and Supplementary table 2 for a detailed description.

Graphical Interface

The most important component of the framework is the user-friendly Graphical Interface (Fig. 3), which allows for characterization and exploration of all single unit metrics through a rich set of high-quality built-in plots, interaction modes, cell grouping, cross-level pointers, and filters. User-defined numbers of plots can be selected at any time and replaced on the screen instantaneously. In the typical layout, the top row displays population-level representations, and the bottom row single-cell features (Fig. 3A). Any neuron or multiple neurons can be clicked upon and all other features of the selected neurons are automatically updated in the other plots. For easy navigation and selection, the left mouse click links to the selected cell(s) and the right mouse click selects the cell(s) from all the plots. These selected groups can be displayed alone or highlighted and superimposed against all data in the same session, multiple sessions, or the entire database. Clusters of neurons of interest can be selected by drawing polygons with the mouse cursor, and the features of the selected groups will be updated in all other display windows. Multiple group selections are also possible for both visualization and statistical comparison. Flexibility is assisted by self-explanatory side panels, including a custom plot panel, color group panel, display settings panel, and legend (left side) and single-cell dimensions, navigation panel, cell assignment panel, tags, and a table with metrics (right side). A text field allows for custom text filters and a message logging. Group plotting options include 2D, 3D, raincloud plot, t-SNE, and double histogram, each plotted either on linear or logarithmic scales (Fig. 3E).

Examples of the flexible operation of the graphical interface module are illustrated in Fig. 4 and described in more detail in Supplementary video 1. Here we begin with motives of monosynaptically connected clusters of neurons from the hippocampal CA1 area, as provided by the Processing Module (Fig. 4A). An example sub-network of connected neurons is highlighted in panel B with a selected single neuron (arrow) to be characterized. selected level one, two, and three descriptors of the neuron are displayed in panels C to G, respectively. In several panels, the descriptors of the selected neuron are shown against all other neurons. Left mouse clicking any neuron will update all the panels, allowing quick screening and qualitative evaluation of multiple features of each inspected neuron. Neurons of interest can be marked for further quantitative comparisons. Next, level one to three descriptions can be compared with paradigm-specific features of the selected neuron(s), such as place field, trial-by-trial variability of firing patterns, travel direction firing specificity, spike phase precession relative to theta oscillation cycles, and multiple other features predefined by the experimenter. During the data mining process, unexpected features and outliers may be noted, instabilities of neurons (‘drifts’) can be recognized and artifacts identified. Such experimenter-supervised judgments are also essential for evaluating the quality of quantified data processing and estimating potential single neuron-unique features that might drive population statistics.

Several tests were performed to characterize the speed performance of CellExplorer under various conditions, testing display features, and computation of various features. The results are summarized in Supplementary Fig. 5.

Figure 4. Data exploration example. A. Connectivity graph with monosynaptic modules found across multiple datasets. Cells are color-coded by their putative cell types (pyramidal cells in red, narrow interneurons in blue and wide interneurons in cyan). B. Highlighted monosynaptic module with single pyramidal cell highlighted (arrow). C. First level descriptors: Auto-correlogram, average waveform (top row), the ISI distribution of the population with the selected cell in thick black, and the physical location of the cells relative to the multi-shank silicon probe. D. Firing rate across time for the population, each cell is normalized to its peak rate. The session consists of three behavioral epochs: pre-behavior sleep, behavior (track running), and post-behavior sleep (boundaries shown with dashed lines). E. Theta phase distribution for all neurons recorded in the same session (red, pyramidal cells; blue, interneurons) during locomotion with the selected cell highlighted (black line). F. Average ripple for multiple sites of a shank. The site of the selected neuron is highlighted (dashed black line). The polarity of the average sharp wave is used to determine the position of the cell relative to the pyramidal layer in CA1. G. Ripple wave-triggered PSTH for the selected cell aligned to the ripple peak. H. Trial-wise raster for the selected cell in a maze. I. The average firing rate of the cell across trials. J. Spike raster showing the theta phase relationship to the spatial location of the animal. Each of the three place fields shows phase precession.

Value of large inter-laboratory data sets

While progress in discovery science often depends on the investigator-unique approach to novel insights, standardization of data processing and screening is essential in fields where ‘big data’ generation is achieved through collaborative efforts. This applies to the current effort to quantitatively relate physiology-based and genetically classified cell ‘types’ (Klausberger and Somogyi, 2008; McBain and Fisahn, 2001; Rudy et al., 2011). In each experiment, typically only one or a limited number of neuron types can be identified. Yet, combining data sets from numerous experiments and different laboratories can generate physiological descriptors grounded by optogenetics and other ‘ground truth’ data.

Fig. 5 illustrates the feasibility and utility of this approach. Level 1-3 descriptors of neurons recorded from the same brain region and layer can be combined from multiple experiments and contrasted to the data quality of units recorded in a single session. An ever-growing data set allows for more reliable modality separation and characterization of neuron types. For example, the initial divisions of neurons into putative pyramidal cells, interneurons, and unclassified cells can be further refined by quantifying monosynaptic connections, increasing confidence of pyramidal cell–interneuron separation as well as identifying subsets of the unclassified group as interneurons (Mizuseki et al., 2011; Petersen and Buzsáki, 2020; Peyrache et al., 2015; Stark et al., 2013). Combining extracellular spiking with intracellular recordings can further help determine cell the excitatory or inhibitory identity of neurons (Radosevic et al., 2019). Single neurons identified by opto-tagging or other direct means (Ciocchi et al., 2015; Klausberger and Somogyi, 2008; Royer et al., 2012; Senzai et al., 2019; Stark et al., 2012; Zhang et al., 2013; Roux and Buzsáki, 2015) can be used to examine and contrast level 1-3 features of initially classified neurons into further types (Fig 5.B). In turn, such ‘ground truth’ data may offer further clues for including unrecognized brain regions and layer-distinct physiological features of single neurons for more effective classification (Senzai and Buzsáki, 2017). An expected outcome is that growing datasets containing ground truth-verified neurons will allow that in future experiments multiple neuron types recording in the same animal can be reliably identified by physiological descriptors only.

Data sets obtained from different brain areas, different electrode types, and drug conditions can be compared. t-Distributed Stochastic Neighbor Embedding (t-SNE) plots can highlight inconsistencies and differences across recording sessions, identify important regional and layer-specific differences, and alert for interspecies characteristics (Fig. 5).

Figure 5. Ground truth data, reference data, and opto-tagging. A. Distribution of putative cell types (3811 cells), including their projections determined via spike-transmissions CCG curves. B. 407 ground truth cells spanning PV (184), SST (115), Pyramidal cells (44), Axo axonic (35), VGAT (15) and VIP cells (14) projected on the same population of neurons as in A. C. Single session (dots) compared with reference data from 30 reference sessions (shaded zones). D. Opto-tagged data can be processed and curate directly in CellExplorer. E. Example of PSTH of a PV-expressing cell. Raster plot and average responses to light pulses visualized in CellExplorer. F. The CellExplorer framework allows for sharing ground truth and reference data directly with the end-user. End users can upload their ground truth data to the CellExplorer GitHub repository for communal sharing.

Open-source database

CellExplorer takes advantage of web-based resources (Chon et al., 2019; Petersen et al., 2018)  atlas.brain-map.org, buzsakilab.com/wp/database) for discovering, viewing, and comparing the physiological features of single neurons. Data sharing allows building large data banks for discovery science, cross-laboratory interactions, and reproducibility control.

# DISCUSSION

We have developed CellExplorer, a transparent, open-source, MATLAB-based resource for characterizing single neurons and neuron types, using physiological features. The CellExplorer platform enables visualization and analysis for users without the need to write code. Its modular format allows for fast and flexible comparisons of a large battery of preprocessed physiological characteristics of single neurons and their interactions with other neurons as well as their correlation with experimental variables. Code is publicly available on GitHub for users to download and use the same standardized processing module on their local personal computers (Windows, OS X, and Linux). CellExplorer offers step-by-step online tutorials for first-time users. It is linked to the Allen Institute reference atlas (Chon et al., 2019), https://atlas.brain-map.org/) and can be expanded to include other online resources that provide annotated data on putative neuron types. The open-source design of CellExplorer permits the optimization of current standardized components tested against an ever-growing community-contributed database and will accelerate linking genetically identified neuron types with their physiological properties in the intact brain.

Multiple-level characterization and classification of single neurons

To correctly interpret neuron firing-behavior/cognition relationships, numerous controls are needed to rule out or reduce the potential contribution of spurious variables. The Processing Module generates a battery of useful metrics. In addition to the first-level description of the biophysical and physiological characteristics of single neurons, it computes brain state-dependent firing rates, interspike interval variation, and relationships between single neurons and spiking activity of the population and LFP (second level). When spontaneous (control) behaviors are also available, it describes the relationship between single neuron firing patterns and routine behavioral parameters, such as immobility, walking, running speed, respiration, and pupil diameter (third level). These third-level descriptors may assist inappropriately attributing spiking activity to inferred behavior, such as perception or cognition. Because these 3-level descriptors are independent of particular experimental paradigms, they can be used as benchmarks for assessing consistencies across experiments performed by different investigators in the same laboratory or across laboratories. Concatenating numerous data sets obtained from the same brain regions and layers will create a continuously growing data bank. In turn, these data-rich sets may allow identifying and quantifying reliable boundaries among putative clusters and suggest inclusion and exclusion of parameters for a more refined separation of putative neuronal classes. Sets from different brain regions can be readily compared and differences recognized.

Although several statistical tests are available in CellExplorer, it is not meant to be a substitute for rigorous quantification. Instead, it is designed as a tool for facilitating interpretation and discovery. It is a complementary approach to dimensionality reduction and population analysis methods. Because assemblies of neurons consist of highly unequal partners (Buzsáki and Mizuseki, 2014), knowledge about the neuron-specific contribution to population measures is critical in many situations (Nicolelis and Lebedev, 2009). Such inequality may stem from unknowingly lumping neurons of different classes together into a single type and because even members of the same type belong to broad and skewed distribution and may contribute to different aspects of the experiment (Grosmark and Buzsáki, 2016).

Two key features make the CellExplorer platform highly efficient: flexibility and speed. Flexibility is provided by the numerous parameters as outputs of the Processing Module (metrics listed in supplementary table 2). High speed is achieved by using pre-computed metrics and limiting the computation time in the user interface. A caveat is that online alteration of the plots and metrics is not allowed. Yet, such changes can be performed by working with the raw spike data. Multiple features of single neurons, displayed on the same screen, can be compared. Moving from one neuron to the next requires only a mouse click. These features can be compared separately or superimposed against another neuron, all neurons in a session, or the entire database. Unexpected features can be noted, obvious artifacts can be deleted. When unusual sets are discovered in any display, all other features of the same set can be rapidly compared and contrasted to other sets. Neuron clouds can be selected by drawing polygons around them and regrouped in any -user-defined configuration. Inspection of data sets containing even several thousand neurons (Steinmetz et al., 2019) is realistic because minimal computing time is required in the graphical interface and because in most conditions only small subsets need individual inspection and quality control.

Various classification schemes have been developed to assign extracellular spikes to putative pyramidal cells, interneurons, and their putative subtypes, based on a variety of physiological criteria. These include waveform features, firing rate statistics in different brain states, embeddedness in various population activities, firing patterns characterized by their autocorrelograms, and putative monosynaptic connections to other neurons (Barthó et al., 2004; Csicsvari et al., 1999; Fujisawa et al., 2008; Mizuseki et al., 2009; Okun et al., 2015; Sirota et al., 2008). Increasingly larger data sets will likely improve such physiology-based classification. Yet, the ‘ground truth’ for these classifying methods is largely missing. Optogenetic tagging (Boyden et al., 2005) offers such grounding by connecting putative subtypes based on physiologically distinct features to their molecular identities. Because in a single animal only one or few neuron types can be tagged optogenetically or by other direct methods (Fosque et al., 2015; Klausberger and Somogyi, 2008), refinement of a library of physiological parameters should be conducted iteratively, so that in subsequent experiments the various neuron types can be recognized reliably by using solely physiological criteria (English et al., 2017; Royer et al., 2012; Senzai and Buzsáki, 2017, 2017; Roux and Buzsáki, 2015). In turn, knowledge about the molecular identity of the different neuronal components of a circuit can considerably improve the interpretation of correlational observations provided by large-scale extracellular recordings.

Data sharing

Above, we described one of the many possible examples that can benefit from large databases. Currently, tens to hundreds of thousands of pre-processed neurons exist across laboratories in different brain regions, which can be identically streamlined by the Processing Module, and displayed and compared in the same coordinate system. Our laboratory welcomes shared data sets from other research groups for enhancement and comparison with our publicly-accessible database (buzsakilab.com/wp/database). The single prerequisite for quantitative comparison of data across laboratories is to make wideband data available (≤ 3 Hz to ≥ 8 kHz; ideally ≥ 20 kHz sampling rate) so that all data are processed the same way.

Through our web resource (Petersen et al., 2018), we host > 1,000 publicly shared data sets of long (4 to 24 hrs), large-scale recordings of single units from multiple brain structures, including the hippocampus, entorhinal, prefrontal, somatosensory, and visual cortices, thalamus, amygdala, and septum (buzsakilab.com/wp/database). Long-recordings have the advantages of defining brain state-dependent characteristics of neurons, such as their firing rates and patterns during waking and sleep, unmasking the ‘hidden’ or a relatively silent majority of neurons (Mizuseki and Buzsáki, 2013; Shoham et al., 2006) and discovering their connectivity patterns (English et al., 2017). These data already provide benchmarks assessing the reliability of initial neuron classification into the broad groups of pyramidal cells and interneurons, many of which are identified physiologically by their monosynaptic connections. They also offer normative data about spikes features, firing rates, and spike dynamics. These features can serve as benchmarks for comparison with data collected in any other laboratory.

Development and availability

Development takes place in a public code repository at github.com/petersenpeter/CellExplorer. All examples in this article have been calculated with the pipeline and plotted with CellExplorer. Extensive documentation, including installation instructions and tutorials, is hosted at CellExplorer.org. CellExplorer is available for MATLAB 2017B and forward, and for the operating systems Windows, OS X, and Linux. More information can be found at CellExplorer.org.

ACKNOWLEDGEMENTS

We thank Sam McKenzie, Mihály Vöröslakos, Michelle Hernandez, Thomas Hainmueller, for helping with documentation or implementation of CellExplorer code. Further, we thank Sam McKenzie, Daniel English, Roman Huszar, and Yuta Senzai for providing ground truth datasets. Finally, thanks go to Thomas Hainmueller, Manuel Valero, Antonio Hernandez-Ruiz, Viktor Varga, and Omid Yaghmazadeh for comments on the manuscript. Supported by U19 NS107616, U19 NS104590, R01 MH122391, and The Lundbeck Foundation.

# REFERENCES

Barlow, H.B. (1972). Single Units and Sensation: A Neuron Doctrine for Perceptual Psychology? Perception 1, 371–394.

Barthó, P., Hirase, H., Monconduit, L., Zugaro, M., Harris, K.D., and Buzsáki, G. (2004). Characterization of neocortical principal cells and interneurons by network interactions and extracellular features. J. Neurophysiol. 92, 600–608.

Boyden, E.S., Zhang, F., Bamberg, E., Nagel, G., and Deisseroth, K. (2005). Millisecond-timescale, genetically targeted optical control of neural activity. Nature Neuroscience 8, 1263–1268.

Buzsáki, G. (2004). Large-scale recording of neuronal ensembles. Nature Neuroscience 7, 446–451.

Buzsáki, G., and Mizuseki, K. (2014). The log-dynamic brain: how skewed distributions affect network operations. Nat Rev Neurosci 15, 264–278.

Buzsáki, G., Stark, E., Berényi, A., Khodagholy, D., Kipke, D.R., Yoon, E., and Wise, K. (2015). Tools for probing local circuits: high-density silicon probes combined with optogenetics. Neuron 86, 92–105.

Chon, U., Vanselow, D.J., Cheng, K.C., and Kim, Y. (2019). Enhanced and unified anatomical labeling for a common mouse brain atlas. Nature Communications 10, 1–12.

Chung, J.E., Magland, J.F., Barnett, A.H., Tolosa, V.M., Tooker, A.C., Lee, K.Y., Shah, K.G., Felix, S.H., Frank, L.M., and Greengard, L.F. (2017). A Fully Automated Approach to Spike Sorting. Neuron 95, 1381-1394.e6.

Ciocchi, S., Passecker, J., Malagon-Vina, H., Mikus, N., and Klausberger, T. (2015). Selective information routing by ventral hippocampal CA1 projection neurons. Science 348, 560–563.

Csicsvari,  tJozsef, Hirase, H., Czurkó, A., Mamiya, A., and Buzsáki, G. (1999). Fast Network Oscillations in the Hippocampal CA1 Region of the Behaving Rat. J. Neurosci. 19, RC20–RC20.

Csicsvari, J., Henze, D.A., Jamieson, B., Harris, K.D., Sirota, A., Barthó, P., Wise, K.D., and Buzsáki, G. (2003). Massively Parallel Recording of Unit and Local Field Potentials With  Silicon-Based Electrodes. Journal of Neurophysiology 90, 1314–1323.

English, D.F., McKenzie, S., Evans, T., Kim, K., Yoon, E., and Buzsáki, G. (2017). Pyramidal Cell-Interneuron Circuit Architecture and Dynamics in Hippocampal Networks. Neuron 96, 505-520.e7.

Fosque, B.F., Sun, Y., Dana, H., Yang, C.-T., Ohyama, T., Tadross, M.R., Patel, R., Zlatic, M., Kim, D.S., Ahrens, M.B., et al. (2015). Labeling of active neural circuits in vivo with designed calcium integrators. Science 347, 755–760.

Fujisawa, S., Amarasingham, A., Harrison, M.T., and Buzsáki, G. (2008). Behavior-dependent short-term assembly dynamics in the medial prefrontal cortex. Nature Neuroscience 11, 823–833.

Grosmark, A.D., and Buzsáki, G. (2016). Diversity in neural firing dynamics supports both rigid and learned hippocampal sequences. Science 351, 1440–1443.

Hazan, L., Zugaro, M., and Buzsáki, G. (2006). Klusters, NeuroScope, NDManager: A free software suite for neurophysiological data processing and visualization. Journal of Neuroscience Methods 155, 207–216.

Kepecs, A., and Fishell, G. (2014). Interneuron cell types are fit to function. Nature 505, 318–326.

Klausberger, T., and Somogyi, P. (2008). Neuronal Diversity and Temporal Dynamics: The Unity of Hippocampal Circuit Operations. Science 321, 53–57.

McBain, C.J., and Fisahn, A. (2001). Interneurons unbound. Nat. Rev. Neurosci. 2, 11–23.

McGinley, M.J., David, S.V., and McCormick, D.A. (2015). Cortical Membrane Potential Signature of Optimal States for Sensory Signal Detection. Neuron 87, 179–192.

Mizuseki, K., and Buzsáki, G. (2013). Preconfigured, Skewed Distribution of Firing Rates in the Hippocampus and Entorhinal Cortex. Cell Reports 4, 1010–1021.

Mizuseki, K., Sirota, A., Pastalkova, E., and Buzsáki, G. (2009). Theta Oscillations Provide Temporal Windows for Local Circuit Computation in the Entorhinal-Hippocampal Loop. Neuron 64, 267–280.

Mizuseki, K., Diba, K., Pastalkova, E., and Buzsáki, G. (2011). Hippocampal CA1 pyramidal cells form functionally distinct sublayers. Nature Neuroscience 14, 1174–1181.

Nicolelis, M.A.L., and Lebedev, M.A. (2009). Principles of neural ensemble physiology underlying the operation of brain-machine interfaces. Nature Reviews Neuroscience 10, 530–540.

Okun, M., Steinmetz, N.A., Cossell, L., Iacaruso, M.F., Ko, H., Barthó, P., Moore, T., Hofer, S.B., Mrsic-Flogel, T.D., Carandini, M., et al. (2015). Diverse coupling of neurons to populations in sensory cortex. Nature 521, 511–515.

Pachitariu, M., Steinmetz, N.A., Kadir, S.N., Carandini, M., and Harris, K.D. (2016). Fast and accurate spike sorting of high-channel count probes with KiloSort. In Advances in Neural Information Processing Systems 29, D.D. Lee, M. Sugiyama, U.V. Luxburg, I. Guyon, and R. Garnett, eds. (Curran Associates, Inc.), pp. 4448–4456.

Petersen, P.C., and Berg, R.W. (2016). Lognormal firing rate distribution reveals prominent fluctuation–driven regime in spinal motor networks. ELife 5, e18805.

Petersen, P.C., and Buzsáki, G. (2020). Cooling of medial septum reveals theta phase lag coordination of hippocampal cell assemblies. BioRxiv 2019.12.19.883421.

Petersen, P.C., Hernandez, M., and Buzsáki, G. (2018). Public electrophysiological datasets collected in the Buzsaki Lab. https://buzsakilab.com/wp/public-data/ http://doi.org/10.5281/zenodo.3629881 (Zenodo).

Peyrache, A., Lacroix, M.M., Petersen, P.C., and Buzsáki, G. (2015). Internally organized mechanisms of the head direction sense. Nat Neurosci 18, 569–575.

Quiroga, R.Q., Nadasdy, Z., and Ben-Shaul, Y. (2004). Unsupervised Spike Detection and Sorting with Wavelets and Superparamagnetic Clustering. Neural Computation 16, 1661–1687.

Radosevic, M., Willumsen, A., Petersen, P.C., Lindén, H., Vestergaard, M., and Berg, R.W. (2019). Decoupling of timescales reveals sparse convergent CPG network in the adult spinal cord. Nat Commun 10, 2937.

Roux, L., and Buzsáki, G. (2015). Tasks for inhibitory interneurons in intact brain circuits. Neuropharmacology 88, 10–23.

Royer, S., Zemelman, B.V., Losonczy, A., Kim, J., Chance, F., Magee, J.C., and Buzsáki, G. (2012). Control of timing, rate and bursts of hippocampal place cells by dendritic and somatic inhibition. Nature Neuroscience 15, 769–775.

Rudy, B., Fishell, G., Lee, S., and Hjerling‐Leffler, J. (2011). Three groups of interneurons account for nearly 100% of neocortical GABAergic neurons. Developmental Neurobiology 71, 45–61.

Schmitzer-Torbert, N., Jackson, J., Henze, D., Harris, K., and Redish, A.D. (2005). Quantitative measures of cluster quality for use in extracellular recordings. Neuroscience 131, 1–11.

Senzai, Y., and Buzsáki, G. (2017). Physiological Properties and Behavioral Correlates of Hippocampal Granule Cells and Mossy Cells. Neuron 93, 691-704.e5.

Senzai, Y., Fernandez-Ruiz, A., and Buzsáki, G. (2019). Layer-Specific Physiological Features and Interlaminar Interactions in the Primary Visual Cortex of the Mouse. Neuron 101, 500-513.e5.

Shoham, S., O’Connor, D.H., and Segev, R. (2006). How silent is the brain: is there a “dark matter” problem in neuroscience? J Comp Physiol A 192, 777–784.

Sirota, A., Montgomery, S., Fujisawa, S., Isomura, Y., Zugaro, M., and Buzsáki, G. (2008). Entrainment of neocortical neurons and gamma oscillations by the hippocampal theta rhythm. Neuron 60, 683–697.

Stark, E., Koos, T., and Buzsáki, G. (2012). Diode probes for spatiotemporal optical control of multiple neurons in freely moving animals. J. Neurophysiol. 108, 349–363.

Stark, E., Eichler, R., Roux, L., Fujisawa, S., Rotstein, H.G., and Buzsáki, G. (2013). Inhibition-Induced Theta Resonance in Cortical Circuits. Neuron 80, 1263–1276.

Steinmetz, N.A., Zatka-Haas, P., Carandini, M., and Harris, K.D. (2019). Distributed coding of choice, action and engagement across the mouse brain. Nature 576, 266–273.

Yger, P., Spampinato, G.L., Esposito, E., Lefebvre, B., Deny, S., Gardella, C., Stimberg, M., Jetter, F., Zeck, G., Picaud, S., et al. (2018). A spike sorting toolbox for up to thousands of electrodes validated with ground truth recordings in vitro and in vivo. ELife 7, e34518.

Zhang, S.-J., Ye, J., Miao, C., Tsao, A., Cerniauskas, I., Ledergerber, D., Moser, M.-B., and Moser, E.I. (2013). Optogenetic Dissection of Entorhinal-Hippocampal Functional Connectivity. Science 340.

# SUPPLEMENTARY SECTION

Supplementary figure 1: Datatypes. The data structure. A detailed description is available online at CellExplorer.org/datastructure/data-structure-and-format. session, spikes, cell_metrics, trials are defined data types, while behavior, firingRateMap, events, manipulation, timeseries, states, channelinfo are data containers.

Supplementary Figure 2. Flow charts. A) Generating the metadata structure for a recording session. B) Running the processing pipeline. C) Running the CellExplorer module for manual curation and exploration. CellExplorer data structures are shown in yellow, MATLAB functions in green, and the input data in grey. Input from the Buzsáki lab database is shown in purple (buzsakilab.com/wp/database).

Supplementary Figure 3. The various single-cell plots. Most single cell data-visualizer have three representations: single neurons (with neuronal connections highlighted for a subset of the plots), all neurons (absolute or normalized representations), and an image representation (normalized data, with selected cell highlighted by a white line). A. Waveform representations: waveform of a selected single neuron, waveforms of all neurons (z-scored), and their image representation. The white line in the image representation corresponds to the selected neuron. B. Autocorrelograms (ACGs) for the single neuron, ACGs for all neurons and their image representation. C. ACGs on a log scale (single, all, image).  D, E. Interspike interval distributions (ISIs) on a log scale (single, all image) for two different normalizations (D, rate (Hz); E, occurrence). F. Theta phase spike histogram for the single interneuron (black line) and those of pyramidal neurons monosynaptically connected to the interneurons (blue lines; left) and all neurons in the same session (middle and right panels). G. Firing rate map for a pyramidal cell. Session average (left) and trial-wise heatmap. H. Connectivity graph showing all monosynaptic modules in the dataset. A module is highlighted and enhanced (top right). I. Physical location of neurons recorded in the same animal using trilateration. Eight-shank silicon probe recording (8 sites on each shank). Red, pyramidal cells. Blue, interneurons. Monosynaptic connections between two pyramidal cells and a target interneuron are also shown (blue lines) J. Average waveform across channels of the single interneurons shown in most panels. A-F, H-J: a narrow interneuron, G: Spatial firing rate of a pyramidal cell on a linear track.

Supplementary Figure 4. Population data plots. Top row: The three standard representations: custom plot (A), classic representation (B), and t-SNE plot (C). Bottom row: The custom plot has 3 further data representations: a 3-dimensional plot with custom marker size (D), 2D plot with marginal histograms (E), and one-dimensional raincloud plots (F), combining 1D scattered neurons with error bars histogram and KS significance test (line thickness represent significance levels). Color-coded according to cell types: pyramidal cell (red), narrow interneuron (blue), wide interneuron (cyan).

Supplementary figure 5. Benchmarks of the CellExplorer user interface (UI). A. UI display times when switching between units for the three layouts shown in figure 3B (approximately 110 ms for layout 1+3; blue lines. 180 ms for layout 3+3; green lines) and 290 ms (layout 3+6; in red), respectively. Dark gradient colored lines (dark red, green,  and blue) indicate where there were no limits on the number of traces plotted for single-cell plots, and the light gradient lines show screen update times with a maximum of 2000 random traces. B. Display times for single-cell plots, quantified by the number of cells displayed. The plots contributing most to an increased display time are the plots with trace representations for each cell (ACGs, ISIs, waveforms, ISIs, theta phase) and the connectivity graph. By default, a maximum of 2000 traces is drawn capping the processing time below ~80 ms for all plots except the connectivity graph. C. Benchmarking of cell metrics file readings. On average, 230 cells can be loaded per second quantified across 180 sessions with various cell count (red dots and linear fit in red). By storing the data on a local SSD, the loading time could be decreased and attain cell loading above 500 cells per second.

Supplementary figure 6. Exploration and comparison of metrics and cells across, species, subjects and brain regions. A. Distributions of spike amplitudes and waveform width (quantified by the trough to peak metrics) for the three groups from multiple CA1 recording sessions. Note inverse relationship between spike amplitude and waveform for putative interneurons. B-D. t-SNE representations of putative cell types (B), species (C, rat, and mouse in magenta and red, respectively) and subjects (D, colors scaled across subjects) for hippocampal neurons. E-I: Comparison of spike features of neurons recorded from CA1 pyramidal cells and visual cortex pyramidal cells. Significant differences are observed across several basic metrics, including CV2 (E), burst index (F), trough-to-peak (G), waveform asymmetry (H), and waveform peak voltage (I).

| Functions | Description |
| --- | --- |
| sessionTemplate | A template script which automatically extracts and imports relevant metadata |
| gui_session | A graphical user interface (GUI) for manual inspection and entry of metadata |
| ProcessCellMetrics | The processing module |
| CellExplorer | The main graphical interface of CellExplorer |
| CellExplorer_Preferences | Preference file for the graphical interface |
| gui_MonoSyn | GUI for manual curation of monosynaptic connections |
| gui_DeepSuperficial | GUI for manual curation of the depth assignment of neurons based on depth-related changes of sharp-wave-ripples (Mizuseki et al., 2011) |
| loadCellMetricsBatch | Batch loading script for combining cell_metrics structs across sessions |
| loadCellMetrics | Script for loading cell_metrics with built-in common text filters (putative cell type, brain region, synaptic effect, label, animal, tags, groups, etc.) |

Supplementary Table 1. Primary MATLAB functions of the CellExplorer framework.

Supplementary Table 2: Cell metrics. An incomplete list of the standard cell metrics. The full list is available online at CellExplorer.org/datastructure/standard-cell-metrics

Supplementary video 1

The supplementary video is available on YouTube and at CellExplorer.org.

TUTORIALS

A general tutorial on the full pipeline is described below. There are many more detailed tutorials online, covering: the generation of the metadata struct, the manual curation process, generating spike raster plots, the manual curation process of monosynaptic connections, performing opto-tagging, using ground truth data, export figure, and many other topics.

Tutorials are available online at CellExplorer.org/tutorials/tutorials

General tutorial

This tutorial shows you the full processing pipeline, from generating the necessary session metadata using the template, running the processing pipeline, opening multiple sessions for manual curation in CellExplorer, and finally using the cell_metrics for filtering cells, by two different criteria. The tutorial is also available as a Matlab script: (tutorials/CellExplorer_Tutorial.m).

- Define the basepath of the dataset to process. The dataset should consist of a basename.dat(a binary raw data file),  a basename.xml (recommended; not required) and spike sorted data.

basepath = '/your/data/path/basename/';

cd(basepath)

- Generate session metadata struct using the template function and display the metadata in a GUI

session = sessionTemplate(basepath, 'showGUI', true);

In the GUI you can put in relevant metadata. Please pay attention to the general, extracellular, and spike sorting tabs and verify all metadata.

- Run the cell metrics pipeline ProcessCellMetrics using the session struct as input

cell_metrics = ProcessCellMetrics('session', session);

- Visualize the cell metrics in CellExplorer

cell_metrics = CellExplorer('metrics', cell_metrics);

- Now you can repeat step 1-4 on a couple of datasets and load them together in CellExplorer, providing several paths

basepaths = {'path/to/session1','path/to/session2'};

cell_metrics = LoadCellMetricsBatch(' basepaths', basepaths);

cell_metrics = CellExplorer('metrics', cell_metrics);

- Curate your cells and save the metrics

- Finally, to incorporate the cell metrics into your analysis you can use the load function that has filters built-in:

  - Get cells that are assigned as Interneuron

cell_metrics_idxs1 = loadCellMetrics('cell_metrics', cell_metrics, 'putativeCellType', {'Interneuron'});

  - Get cells that are has groundTruthClassification as Axoaxonic

cell_metrics_idxs2 = loadCellMetrics('cell_metrics', cell_metrics, 'groundTruthClassification', {'Axoaxonic'});